1. Identification

GHS Product Identifier  	Conclean

Other Names  		902 (Manufacturer’s production code)

Company Name 		HY.GIENE Australia Pty Ltd

Address 			Unit 11, 18-20 Edward Street, OAKLEIGH, VIC  3166

Telephone		(03) 9729 3946

Emergency Contact	1800 616 930

Recommended use	Cleaning Agent for hard surfaces

2. Hazard Identification

Statement of Hazardous Nature

This product is not classified as Hazardous according to the criteria of Safe Work Australia.

NOT a Dangerous Good according to the Australian Dangerous Goods (ADG) Code version 7.

GHS Signal Word (s)	WARNING

Hazard Statement(s)	H315 Causes skin irritation.

H320 Causes eye irritation.

GHS Pictogram (s)

Precautionary statement(s)

Prevention 		P280 Wear protective gloves and eye protection

Response		P353 Rinse skin or shower with water.

P301+P330+P338: IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P305+P351+P338: IF IN EYES: Rinse with water for several minutes.

Remove contact lenses, if present. Continue rinsing.

P332+P313: If skin irritation occurs: Get medical advice.

P337+P313: If eye irritation persists: Get medical advice.

P391: Collect spillage.

P370+P378: Not combustible. Use extinguishing media suited to burning materials.

Storage P233+P234: Keep only in original container. Keep container tightly closed. Disposal P501 Dispose of contents/container to an approved waste disposal plant.

3. Composition/Information on Ingredients

Chemical characterization		Aqueous blend of quaternary ammonium compounds, < 5% and surfactant.

Hazardous ingredients

Name					CAS no.		 	Proportion

Quaternary Ammonium			107028-70-6	 	  LOW

Compounds

KEY: Proportion, (wt %) 		 	V HIGH >60,   HIGH 30 - 60,   MED 10 –29,   LOW 1-9,   V LOW <1

4. First Aid Measures

Ingestion:	Rinse mouth with water. Never give anything by mouth to an unconscious person. Consult a physician.

Skin:	Wash off with soap and plenty of water.

Eye contact	Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

Inhalation	If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

First Aid Facilities	Maintain eyewash fountain and safety shower in work area.

Advice to Doctor	Treat symptomatically. Consult Poisons Information Centre

Other Information	For advice, contact the National Poisons Information Centre

(Phone Australia 13 11 26 and New Zealand 0800 764 766) or a doctor.

5. Fire Fighting Measures

Suitable extinguishing	Use extinguishing media most appropriate for the surrounding fire such as water, foam or dry agent

media 	(carbon	dioxide, dry chemical powder). If safe to do so, move undamaged containers from the fire area. If a significant quantity (>200L) of this product is involved in a fire, call the fire brigade.

Specific hazards arising

from the chemical 	There is no risk of an explosion from this product under normal circumstances if it is involved in a fire. This product is likely to decompose only after heating to dryness, followed by further strong heating.

Precautions in connection Wear SCBA and chemical splash suit. Fully encapsulating, gas-tight suits should be worn for maximum

with fire			protection.

6. Accidental Release Measures

Personal Precautions	Use personal protective equipment. Avoid breathing mist . Ensure adequate ventilation.

Evacuate personnel to safe areas. For personal protection see section 8.

Personal Protection	Wear protective clothing specified for normal operations (see Section 8)

Clean-up Methods-

Small Spillages	Minor spills do not require special clean up measures or emergency procedures. Wear recommended personal protective equipment outlined in Section 8 when containing any spillage.

Large Spillages		Seek expert advice on handling and disposal.

Environmental

Precautions		Prevent further leakage or spillage if safe to do so. Do not let product enter drains or waterways.

7. Handling and Storage

Precautions for Safe 	Product is safe to handle under normal conditions of use.

Handling

Conditions for safe 	Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which

storage 			are opened must be resealed and kept upright to prevent leakage.

Incompatible products	None known

Incompatible materials	None known

8. Exposure Controls/Personal Protection

SWA exposure limits	Exposure limits have not been established by SWA for any significant ingredients in this product.

STEL	(mg/m3)			TWA	(mg/m3)

Name

Quaternary Ammonium		Not Set				Not Set

Compounds

Appropriate engineering 	In industrial situations maintain the concentrations values below the TWA. This may be achieved by

controls			process modification, use of local exhaust ventilation, capturing substances at the source, or other

methods.

Personal Protective 	Final choice of personal protective equipment will depend on individual circumstances and/or

Equipment 	according to risk assessments undertaken.

Respiratory Protection	Usually, no respirator is necessary when using this product.

Eye Protection		The use of safety glasses with side shield protection is recommended when using this product.

Must comply with Australian Standards AS 1337 and be selected and used in accordance with

AS 1336.

Hand Protection		Avoid skin contact when removing gloves from hands, do not touch the gloves outer surface.

Hand protection should comply with AS 2161, Occupational protective gloves - Selection, use and

maintenance.

Footwear		Safety boots in industrial situations is advisory, foot protection should comply with AS 2210,

Occupational protective footwear - Guide to selection, care and use.

Hygiene Measures	Do not eat, drink or smoke in work areas. Wash hands thoroughly after handling this material. Maintain good housekeeping.

9. Physical and Chemical Properties

Appearance		Thin, colourless liquid

Odour			Unscented

Melting Point		 ~0 °C

Boiling Point		~ 100 °C

Flash point 		Not applicable

Vapour Pressure		Not determined

Solubility 	Miscible with water in all proportions.

Specific Gravity		1.0 g/cm3 @ 20 °C

pH			6.5 – 7.5 (as supplied)

Viscosity		Not determined

Percent volatile		> 95 %

Flammability		Non flammable

10. Stability and reactivity

Chemical stability	Stable under normal conditions of use.

Conditions to avoid	No special conditions. Refer to storage conditions in Section 7.

Incompatible materials	None known.

Hazardous decomposition

products 		Carbon monoxide, carbon dioxide and possibly oxides of nitrogen.

Hazardous reactions	None.

Hazardous 		Will not occur.

polymerization

11. Toxicological Information

Local Effects		Irritant: eye, skin.

Target Organs		Eyes and skin.

12. Ecological Information

Ecotoxicity 		Data from 50% quaternary ammonium compound:

Fish Toxicity (Zebra Fish): LC50, 1-10 mg/L

% Biological elimination: 35-70%   [Method: OECD 302B]

Persistence and

degradability 		Biodegradable.

13. Disposal Considerations

Disposal 	The small quantities contained in wash solutions can be handled by conventional sewage, septic and grey water systems. To dispose of large quantities of undiluted product, refer to State Land Waste Management Authority. Containers should be emptied as completely as practical before disposal.

14. Transport information

ADG Code		This product is NOT a Dangerous Good. No special transport conditions are necessary.

15. Regulatory information

AICS			All ingredients listed in the Australian Inventory of Chemical Substances (AICS)

GHS Classification	Skin Irritation Category 2

Eye Irritation Category 2

SUSMP			None allocated

ADG Code		None allocated

16. Other Information

Date of preparation or last

revision of SDS		23 Jan 2025

THIS SDS SUMMARISES OUR BEST KNOWLEDGE OF THE HEALTH AND SAFETY HAZARD INFORMATION OF THIS PRODUCT AND HOW TO SAFELY USE THIS PRODUCT IN THE WORKPLACE.  EACH USER MUST REVIEW THIS SDS IN THE CONTEXT OF HOW THE PRODUCT WILL BE HANDLED AND USED IN THE WORKPLACE.

IF CLARIFICATION OR FURTHER INFORMATION IS NEEDED TO ENSURE THAT AN APPROPRIATE RISK ASSESSMENT CAN BE MADE, THE USER SHOULD CONTACT THIS COMPANY SO WE CAN ATTEMPT TO OBTAIN ADDITIONAL INFORMATION FROM OUR SUPPLIERS.