1. Identification

GHS Product identifier  	Zymo

Other Names  		# 350 (Manufacturer’s production code)

Company Name 		HY.GIENE Australia Pty Ltd

Address 			Unit 11, 18-20 Edward Street, OAKLEIGH, VIC  3166

Telephone		(03) 9729 3946

Emergency Contact 	1800 616 930

Recommended use  	Enzymatic Floor & Hard Surface Cleaner

2. Hazard Identification

Statement of Hazardous Nature

Hazardous according to the criteria of Safe Work Australia.

NOT a Dangerous Good according to the “Australian Code for the Transport of Dangerous Goods by Road & Rail”

GHS classification of

Skin Corrosion/Irritation		Category 2

Serious Eye damage/irritation	Category 2A

Signal Word (s)		WARNING

Hazard Statements		H 315: Causes skin irritation.

H 319: Causes serious eye irritation.

Precautionary Statements

Prevention 		P264: Wash hands after use.

P280: Wear protective gloves.

Response	P305 + P351 + P338: IF IN EYES. Rinse with water for several minutes. Remove contact lenses, if present, and continue rinsing.

P303 + P361 + P353: IF ON SKIN (or hair).  Remove immediately all contaminated clothing. Rinse skin with water.

P304 + P340: IF INHALED. Remove victim to fresh air and keep in a position comfortable for breathing.

Storage None allocated. Disposal None allocated.

3. Composition/information on ingredients

Hazardous ingredients

Name								CAS no.		Proportion

Triethanolamine							102-71-6	    	   LOW

Non-hazardous ingredients to 100%

KEY: Proportion, (wt %) - V HIGH >60,  HIGH 30 - 60,   MED 10 –29,   LOW 1-9,   V LOW <1

4. First-aid measures

Ingestion:	Rinse mouth with water. Never give anything by mouth to an unconscious person.

Skin:	Wash off with soap and plenty of water. Consult a physician if irritation, burning or redness develops.

Eye contact	Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

Inhalation	If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

First Aid Facilities		Normal washroom facilities.

Advice to Doctor	Treat symptomatically.

Other Information	For advice, contact the National Poisons Information Centre (Phone Australia 13 11 26 and New Zealand 0800 764 766) or a doctor.

5. Fire-fighting measures

Suitable extinguishing	Use extinguishing media most appropriate for the surrounding fire such as water, foam or dry agent (carbon

media	dioxide, dry chemical powder).If safe to do so, move undamaged containers from the fire area If a significant quantity (>200L) of this product is involved in a fire, call the fire brigade.

Specific hazards arising

from the chemical 	There is no risk of an explosion from this product under normal circumstances if it is involved in a fire. This product is likely to decompose only after heating to dryness, followed by further strong heating.

Precautions in connection 	Wear SCBA and chemical splash suit. Fully encapsulating, gas-tight suits should be worn for maximum

with fire			protection.

6. Accidental release measures

Personal Precautions	Use personal protective equipment. Ensure adequate ventilation.

Evacuate personnel to safe areas. For personal protection. (see section 8)

Personal Protection	Wear protective clothing specified for normal operations. (see Section 8)

Clean-up Methods:

Small Spillages	Minor spills do not require special clean up measures or emergency procedures. Wear recommended personal protective equipment outlined in Section 8 when containing any spillage.

Large Spillages		Seek expert advice on handling and disposal.

Environmental Precautions	Prevent further leakage or spillage if safe to do so. Do not let product enter drains or waterways.

7. Handling and storage

Precautions for Safe	Make sure that the product does not come into contact with substances listed under “Incompatible Materials”

Handling 	in Section 10.

Conditions for safe storage	Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are

opened must be carefully resealed and kept upright to prevent leakage.

8. Exposure controls/personal protection

Occupational  Exposure Limit

values

STEL				TWA

Name 			mg/m3		ppm		mg/m3		ppm		Footnote

Triethanolamine		    -                	 -		     2		   -

Appropriate engineering 	In industrial situations maintain the concentrations values below the TWA. This may be achieved by

controls			process modification, use of local exhaust ventilation, capturing substances at the source, or other

methods.

Personal Protective 	Final choice of personal protective equipment will depend on individual circumstances and/or according

Equipment		to risk assessments undertaken.

Respiratory Protection	Where ventilation is not adequate, respiratory protection may be required. Avoid breathing mists.

Respiratory protection should comply with AS 1716 - Respiratory Protective Devices and be

selected in accordance with AS 1715 - Selection, Use and Maintenance of Respiratory Protective

Devices. Filter capacity and respirator type depends on exposure levels

Eye Protection		The use of safety glasses with side shield protection is recommended.

Must comply with Australian Standards AS 1337 and be selected and used in accordance with AS 1336.

Hand Protection		Avoid skin contact when removing gloves from hands.

Hand protection should comply with AS 2161, Occupational Protective Gloves

Footwear			Safety footwear in industrial situations is advisory.

Body Protection		Clean clothing or protective clothing should be worn, preferably with and apron. Clothing for protection

against chemicals should comply with AS 3765 Clothing for Protection Against Hazardous Chemicals.

Hygiene Measures		Do not eat, drink or smoke in work areas. Wash hands thoroughly after handling this material. Maintain

good housekeeping.

9. Physical and chemical properties

Appearance		Viscous, green liquid

Odour			Characteristic

Melting Point		 ~0 °C

Boiling Point		~ 100 °C

Flash point 		Not applicable

Vapour Pressure		Not determined

Solubility 	Miscible with water in all proportions.

Specific Gravity		1.1 g/cm3 @ 20 °C

pH			8.5 to 9.0 (as supplied)

Viscosity			~ 300 cPs @ 20 °C

Percent volatile		> 90 %

Flammability		Non flammable

10. Stability and reactivity

Chemical Stability		Stable under normal conditions of use.

Conditions to Avoid	No special conditions. Refer to storage conditions in Section 7.

Incompatible Materials	Oxidising agents and acids.

Hazardous Decomposition

products 			Likely products are carbon dioxide and water.

Possibility of

hazardous reactions	Not expected to occur.

Hazardous Polymerization	Will not occur.

11. Toxicological Information

Acute toxicity

Health effects from the likely routes of exposure:

Inhalation		Not expected to cause respiratory irritation.

Skin			Irritant.

Eye			Serious eye damage.

Ingestion			May be harmful if swallowed. May result in irritation to the gastrointestinal tract.

Target Organs:

STOS – single exposure	No known significant effects or hazards.

STOS – repeated exposure	No known significant effects or hazards.

12. Ecological Information

Ecotoxicity 		100% Biological elimination  [Method: OECD 302B]

Persistence and

degradability 		Biodegradable.

13. Disposal Considerations

Disposal 	The small quantities contained in wash solutions can be handled by conventional sewage, septic and grey water systems. To dispose of large quantities of undiluted product, refer to State Land Waste Management Authority. Containers should be emptied as completely as practical before disposal.

14. Transport information

NOT a Dangerous Good according to the “Australian Code for the Transport of Dangerous Goods by Road o& Rail”

U.N. Number		None allocated

UN proper shipping name	None allocated

Transport hazard class(es) 	None allocated

Hazchem Code		None allocated

Packing Group		None allocated

15. Regulatory information

Regulatory Information	Ingredients listed in the Australian Inventory of Chemical Substances (AICS) and are compliant with NICNAS regulations.

Poisons Schedule		None allocated

16. Other Information

Date of preparation or last

revision of SDS		1st November, 2023

THIS SDS SUMMARISES OUR BEST KNOWLEDGE OF THE HEALTH AND SAFETY HAZARD INFORMATION OF THIS PRODUCT AND HOW TO SAFELY USE THIS PRODUCT IN THE WORKPLACE.  EACH USER MUST REVIEW THIS SDS IN THE CONTEXT OF HOW THE PRODUCT WILL BE HANDLED AND USED IN THE WORKPLACE.

IF CLARIFICATION OR FURTHER INFORMATION IS NEEDED TO ENSURE THAT AN APPROPRIATE RISK ASSESSMENT CAN BE MADE, THE USER SHOULD CONTACT THIS COMPANY SO WE CAN ATTEMPT TO OBTAIN ADDITIONAL INFORMATION FROM OUR SUPPLIERS.